# Tesla Service Advisor

## Top 10 Issues & Correction Codes by Model

# Tesla Service Advisor - Top 10 Issues & Correction Codes by Model

## 🎯 Quick Reference Guide for Common Service Requests

---

## 🚗 MODEL Y - TOP 10 ISSUES

### 1. **Squeaky Suspension (Control Arms/Lateral Links)** ⭐ MOST COMMON

**Issue:** Creaking/squeaking from front suspension, especially when turning or over bumps

**Root Cause:** Worn upper control arms, lateral links, compliance links

Correction Codes:

`3101030001` - Front Upper Control Arm - RH (Remove & Install) - 0.45 hrs

`3101030002` - Front Upper Control Arm - RH (Remove & Replace) - 0.45 hrs

`3101030401` - Front Upper Control Arm - LH (Remove & Install) - 0.54 hrs

`3101030402` - Front Upper Control Arm - LH (Remove & Replace) - 0.54 hrs

`3101030301` - Front Lower Lateral Link (Remove & Install) - 0.42 hrs

**Typical Labor:** 2-3 hours for full suspension refresh

### 2. **Rearview Camera Failure**

**Issue:** Camera shows black screen or error message

**Root Cause:** Camera module failure or connection issue

Correction Codes:

`17220500101` - Camera - Rear Facing (Remove & Install) - 0.12 hrs

`1722040501` - Cover - Camera - Rear Facing (Remove & Install) - 0.06 hrs

`1722040502` - Cover - Camera - Rear Facing (Remove & Replace) - 0.06 hrs

### 3. **Brake Issues / Brake Pad Replacement**

**Issue:** Low brake pads, grinding noises, or soft pedal

**Root Cause:** Normal wear or thermal damage

**Correction Codes:** Search database for "brake rotor" + "front" or "rear" specific codes

### 4. **Window Regulator Malfunction**

**Issue:** Window stuck, moves slowly, or won't open/close

**Root Cause:** Motor or regulator mechanism failure

Correction Codes:

`11500010` - Window Regulator (Recalibrate) - 0.06 hrs

`1150010031` - Window Regulator Motor - Front - LH (Remove & Install) - 0.18 hrs

`1150010032` - Window Regulator Motor - Front - LH (Remove & Replace) - 0.24 hrs

### 5. **Forward Collision Avoidance (Software)**

**Issue:** False alerts, system disabled, or camera errors

**Root Cause:** Software glitch or camera misalignment

**Correction Code:** Requires software diagnostics (check Tesla diagnostic system)

### 6. **Touchscreen/MCU Malfunction**

**Issue:** Unresponsive touchscreen, black screen, or reboot loops

**Root Cause:** Software corruption or MCU module failure

**Correction Code:** Requires MCU diagnostic and potential replacement (labor varies 1-4 hours)

### 7. **Battery/Thermal Management**

**Issue:** Battery warning, reduced power, or overheating

**Root Cause:** Thermal sensor malfunction or cooling system leak

Correction Codes:

`18304353` - MY Thermal Management - Cooling System Test/Adjust - 0.36 hrs

### 8. **Tire/Wheel Alignment/Replacement**

**Issue:** Uneven wear, alignment needed, or tire damage

**Root Cause:** Normal wear or impact damage

Correction Codes:

`34014005` - Seasonal Wheel and Tire Set Swap - 0.50 hrs

`34014505` - Accessory Wheel/Tire Package Assembly - 1.08 hrs

### 9. **Door Latch/Alignment Issues**

**Issue:** Door doesn't latch properly or has large gaps

**Root Cause:** Latch mechanism failure or hinge misalignment

Correction Codes:

`1133020101` - Door Latch - Front - LH (Remove & Install) - 0.66 hrs

`1133020102` - Door Latch - Front - LH (Remove & Replace) - 0.66 hrs

### 10. **Headlight Module Failure**

**Issue:** Headlight out, dim, or intermittent

**Root Cause:** Module failure or connector issue

Correction Codes:

`17400800719` - Headlights (Check Adjustment) - 0.12 hrs

`17130100119` - Headlights (Check Alignment) - 0.18 hrs

---

## 🚗 MODEL 3 - TOP 10 ISSUES

### 1. **Squeaky Suspension Issues** ⭐ MOST COMMON

**Issue:** Creaking/clicking from suspension

**Root Cause:** Control arm wear (similar to Model Y but older design)

**Correction Code:** Search for "control arm" + "Model 3" in database

### 2. **Touchscreen Frozen/Unresponsive**

**Issue:** Screen doesn't respond, black screen, or constant rebooting

**Root Cause:** MCU memory issue or thermal failure

**Correction Code:** MCU reset/replacement (contact Tesla service)

### 3. **Brake System Malfunction**

**Issue:** Brake grinding, pulsing, or weak braking

**Root Cause:** Pad wear or rotor damage

Correction Codes:

`33010100` - Brake Rotor - Front - LH (Measure Runout) - 0.35 hrs

`3301010311` - Brake Rotor - Front (Base Brakes) - LH (Remove & Install) - 0.24 hrs

`3301010312` - Brake Rotor - Front (Base Brakes) - LH (Remove & Replace) - 0.24 hrs

### 4. **Camera/Vision System Fault**

**Issue:** Camera error, autopilot disabled, or vision fault

**Root Cause:** Camera misalignment or module failure

**Correction Code:** Requires MCU diagnostics

### 5. **Window Regulator Failure**

**Issue:** Window stuck, grinds, or won't move

**Root Cause:** Motor or regulator mechanism wear

Correction Codes:

`11500010` - Window Regulator (Recalibrate) - 0.06 hrs

`1150010031` - Window Motor/Regulator Assembly - Front - LH (Remove & Install) - 0.60 hrs

`1150010032` - Window Motor/Regulator Assembly - Front - LH (Remove & Replace) - 0.60 hrs

### 6. **Battery Degradation/Thermal**

**Issue:** Range loss, thermal warnings, or reduced charging

**Root Cause:** Thermal management system failure

**Correction Code:** Requires battery diagnostics

### 7. **Charging Port Issues**

**Issue:** Charger won't connect, loose fitting, or error

**Root Cause:** Port mechanism wear or electrical connector issue

Correction Codes:

`10101810102` - Charge Port Housing (Remove & Replace) - 1.74 hrs

### 8. **Door Panel Gap/Misalignment**

**Issue:** Large gaps between door and fender, or uneven trim

**Root Cause:** Manufacturing issue or hinge wear

Correction Codes:

`1133010041` - Closeout Panel - Door - Front - LH (Remove & Install) - 0.42 hrs

`1133010042` - Closeout Panel - Door - Front - LH (Remove & Replace) - 0.42 hrs

### 9. **Steering Issues**

**Issue:** Steering column noise or binding

**Root Cause:** Column shroud misalignment

Correction Codes:

`1405010121` - Shroud - Upper - Steering Column (Remove & Install) - 0.18 hrs

`1405010122` - Shroud - Upper - Steering Column (Remove & Replace) - 0.18 hrs

### 10. **Air Suspension Problems** (if equipped)

**Issue:** Suspension too low, noisy, or warning lights

**Root Cause:** Compressor failure or air spring leak

**Correction Code:** Search for "compressor" + "air suspension"

---

## 🚗 MODEL S - TOP 10 ISSUES

### 1. **Air Suspension Compressor Failure** ⭐ MOST COMMON

**Issue:** Vehicle sits low, compressor constantly runs, or suspension warning

**Root Cause:** Compressor seal failure or relief valve stuck

Correction Codes:

`31102001` - Compressor - Air Suspension - 1st Gen (Remove & Install) - 1.38 hrs

`31102002` - Compressor - Air Suspension - 1st Gen (Remove & Replace) - 0.72 hrs

`31102301` - Compressor - Air Suspension - 2nd Gen (Remove & Install) - 0.60 hrs

**Typical Labor:** 1-2 hours

### 2. **MCU/Infotainment Malfunction**

**Issue:** Touchscreen black, frozen, or slow

**Root Cause:** MCU module failure (very common on older Model S)

**Correction Code:** MCU replacement required (contact Tesla)

### 3. **Battery/Thermal Management Issues**

**Issue:** Battery thermal warnings or reduced range

**Root Cause:** Thermal sensor malfunction

**Correction Code:** Requires full diagnostic

### 4. **Door/Falcon Door Alignment**

**Issue:** Falcon doors won't open/close or misaligned

**Root Cause:** Motor or hinge mechanism wear

**Correction Code:** Requires mechanical inspection

### 5. **Brake System Degradation**

**Issue:** Soft brake pedal, brake noise, or reduced stopping power

**Root Cause:** Brake fluid contamination or pad wear

Correction Codes:

`33000004` - Diagnosis: Brake System - 0.50 hrs

### 6. **Window Motor/Regulator Failure**

**Issue:** Window stuck, moves slowly, or won't work

**Root Cause:** Motor burnout or regulator wear

Correction Codes:

`11502001` - Window Motor/Regulator Assembly - Front - LH (Remove & Install) - 0.60 hrs

`11502101` - Window Motor/Regulator Assembly - Front - RH (Remove & Install) - 0.60 hrs

### 7. **Steering Column Issues**

**Issue:** Column noise or reduced response

**Root Cause:** Shroud or intermediate shaft wear

Correction Codes:

`14053701` - Shroud - Upper - Steering Column (Remove & Install) - 0.80 hrs

`14053702` - Shroud - Upper - Steering Column (Remove & Replace) - 0.96 hrs

### 8. **Charging System Problems**

**Issue:** Charger malfunction or slow charging

**Root Cause:** Connector corrosion or module failure

Correction Codes:

`18303701` - Hose - Rocker to Charger (Remove & Install) - 0.80 hrs

`18303702` - Hose - Rocker to Charger (Remove & Replace) - 0.96 hrs

### 9. **Light Module Failures**

**Issue:** Headlights or taillights out

**Root Cause:** Module electrical failure

**Correction Code:** Requires lighting module diagnostics

### 10. **Suspension Height Sensor Error**

**Issue:** Suspension warning or height adjustment failure

**Root Cause:** Sensor misalignment or electrical fault

**Correction Code:** Requires suspension diagnostics

---

## 🚗 MODEL X - TOP 10 ISSUES

### 1. **Air Suspension Failure** ⭐ MOST COMMON

**Issue:** Vehicle drops, compressor noise, or system inoperative

**Root Cause:** Same as Model S - compressor or air spring leak

Correction Codes:

`31102001` - Compressor - Air Suspension (Remove & Install) - 1.32 hrs

`31102002` - Compressor - Air Suspension (Remove & Replace) - 1.32 hrs

`31102011` - Bracket - Compressor - Air Suspension (Remove & Install) - 1.14 hrs

### 2. **Falcon Door Malfunction** (Rear Doors)

**Issue:** Door won't open/close, opens when not commanded, or misaligned

**Root Cause:** Motor, hinge, or control module failure

Correction Codes:

`10100303302` - Falcon Door Striker Bracket - LH (Remove & Replace) - 1.92 hrs

`10100303402` - Falcon Door Striker Bracket - RH (Remove & Replace) - 1.92 hrs

### 3. **Brake System Issues**

**Issue:** Brake noise, soft pedal, or warning lights

**Root Cause:** Pad wear or system sensor fault

Correction Codes:

`17402501` - Light - Brake - High Mounted (Remove & Install) - 0.70 hrs

`17402502` - Light - Brake - High Mounted (Remove & Replace) - 0.84 hrs

### 4. **MCU/Software Issues**

**Issue:** Unresponsive screen, freezing, or software crashes

**Root Cause:** MCU module failure

**Correction Code:** MCU replacement (contact Tesla)

### 5. **Battery Thermal Management**

**Issue:** Thermal warnings, reduced range, or overheating

**Root Cause:** Cooling system malfunction

**Correction Code:** Requires thermal diagnostics

### 6. **Window Regulator Problems**

**Issue:** Window stuck or won't operate (all 4 doors)

**Root Cause:** Motor or regulator mechanism failure

Correction Codes:

`11500010` - Window Regulator (Recalibrate) - 0.06 hrs

`11502001` - Window Motor/Regulator Assembly - Front - LH (Remove & Install) - 0.72 hrs

`11502002` - Window Motor/Regulator Assembly - Front - LH (Remove & Replace) - 0.78 hrs

### 7. **Door Handle/Latch Issues**

**Issue:** Interior door handle broken, door won't latch, or exterior handle stuck

**Root Cause:** Handle mechanism wear or latch failure

Correction Codes:

`11401001` - Handle - Interior Release - Door - Front - LH (Remove & Install) - 0.20 hrs

`11401002` - Handle - Interior Release - Door - Front - LH (Remove & Replace) - 0.24 hrs

### 8. **Camera/Vision System**

**Issue:** Camera error, autopilot disabled, or vision fault

**Root Cause:** Camera misalignment or fogging

Correction Codes:

`10200348` - Clean Forward Camera View (Precision Cleaning) - 0.30 hrs

`15050901` - Quad Camera Cover - Lower (Remove & Install) - 0.05 hrs

`15050902` - Quad Camera Cover - Lower (Remove & Replace) - 0.06 hrs

### 9. **Charging Issues**

**Issue:** Slow charging, charger won't connect, or error message

**Root Cause:** Connector corrosion or module failure

Correction Codes:

`10101808802` - Rear Charger Mount Brace (Remove & Replace) - 0.48 hrs

### 10. **Tire/Wheel Alignment**

**Issue:** Uneven tire wear, alignment needed, or wheel damage

**Root Cause:** Suspension wear or impact damage

**Correction Codes:** Search for specific wheel/tire codes for Model X

---

## 🚙 CYBERTRUCK - TOP 10 ISSUES

### 1. **Panel Gaps/Stainless Steel Alignment** ⭐ MOST COMMON

**Issue:** Visible gaps between body panels, misaligned doors, or uneven trim

**Root Cause:** Stainless steel rigidity makes adjustment difficult

**Correction Code:** Search for "panel alignment" or "door adjustment" codes

**Note:** This is a known issue - manual alignment required, labor-intensive

### 2. **Tonneau Cover Malfunction**

**Issue:** Cover won't open/close, stuck, or misaligned

**Root Cause:** Motor or rail misalignment

**Correction Code:** Tonneau cover motor/mechanism codes (search database)

### 3. **Trim/Molding Misalignment**

**Issue:** Trim pieces not flush, sharp edges, or rattling

**Root Cause:** Manufacturing tolerances on stainless steel

**Correction Code:** Trim adjustment/replacement codes

### 4. **Stainless Steel Dent/Finish Repair**

**Issue:** Visible dings, dents, or scratches on body

**Root Cause:** Impact damage (stainless shows everything)

**Correction Code:** Body shop codes for dent/polishing work

### 5. **Charging System Problems**

**Issue:** Charger won't connect or slow charging

**Root Cause:** Port connector issue

Correction Codes:

`1519023121` - Wireless Charger - Center Console (Remove & Install) - 0.12 hrs

`1519023122` - Wireless Charger - Center Console (Remove & Replace) - 0.12 hrs

### 6. **Brake Issues**

**Issue:** Brake noise, reduced stopping power, or warning lights

**Root Cause:** Brake pad wear or rotor damage

Correction Codes:

`33000004` - Diagnosis: Brake System - 0.50 hrs

`33010100` - Brake Rotor - Front - LH (Measure Runout) - 0.35 hrs

### 7. **Window Regulator Failure**

**Issue:** Window stuck, grinds, or won't move

**Root Cause:** Motor failure or regulator wear

Correction Codes:

`1150040011` - Window Regulator - Front Door - LH (Remove & Install) - 0.36 hrs

`1150040012` - Window Regulator - Front Door - LH (Remove & Replace) - 0.36 hrs

### 8. **Suspension/Air System**

**Issue:** Suspension too low, compressor noise, or system fault

**Root Cause:** Compressor or air spring failure

Correction Codes:

`3110008011` - Compressor - Air Suspension (Remove & Install) - 0.84 hrs

`3110008012` - Compressor - Air Suspension (Remove & Replace) - 0.84 hrs

### 9. **Touchscreen/Electronics**

**Issue:** Unresponsive screen, black display, or software issues

**Root Cause:** MCU failure or software glitch

**Correction Code:** MCU diagnostics/replacement (contact Tesla)

### 10. **Door/Handle Issues**

**Issue:** Door alignment, handle broken, or latch failure

**Root Cause:** Cybertruck geometry or handle mechanism wear

**Correction Code:** Door adjustment/replacement codes

---

## 📊 QUICK LABOR ESTIMATES

| Issue | Avg Labor | Cost Level |

|-------|-----------|-----------|

| Window Regulator | 0.25-0.75 hrs | Low ($) |

| Brake Pad/Rotor | 0.35-1.0 hrs | Low-Mid ($$) |

| Camera Replacement | 0.12 hrs | Low ($) |

| Door Latch | 0.66 hrs | Low-Mid ($$) |

| Air Suspension Compressor | 0.6-1.4 hrs | High ($$$) |

| Falcon Door Repair | 1.9+ hrs | Very High ($$$$) |

| MCU/Touchscreen | 1-4 hrs | Very High ($$$$) |

| Suspension (Full Refresh) | 2-3 hrs | Very High ($$$$) |

| Panel Alignment | 1-3 hrs | High ($$$ - $$$) |

---

## ✅ HOW TO USE THIS GUIDE

1. **Customer arrives** with issue

2. **Find issue in your model section**

3. **Note the correction codes**

4. **Look up codes in your database** for exact labor hours

5. **Quote the job** to customer

6. **Log the service code** in Tesla's system

---

**Last Updated:** April 29, 2026

**Database:** 33,394 Tesla correction codes (all models)

**Status:** Ready for production use